名称：结构胶粘剂T冲击剥离长度测定仪

结构胶粘剂冲击剥离测定仪    结构胶粘剂冲击剥离

型号：CJ-900

一、产品简介：

结构胶粘剂T冲击剥离长度测定仪符合结构胶粘剂T冲击剥离长度测定方法及评定标准。采用自由落体式冲击剥离试验装置，适用于室温固化结构胶粘剂韧性重要标志——T冲击剥离长度的测定。同时也适用于抗震设防区建筑加固所使用结构胶粘剂的韧性要求，可按本标准进行测试与合格评定。

product introduction:
The structural adhesive T impact peeling length measuring instrument conforms to the measuring method and evaluation standard of structural adhesive T impact peeling length. The free fall type impact peeling test device is suitable for the determination of the T impact peeling length of the room temperature curing structural adhesive. It is also applicable to the toughness requirements of structural adhesives used in building reinforcement in seismic fortification areas. It can be tested and qualified according to this standard.

二、原理

以一对软钢薄片胶接成T冲击剥离试样，在规定的条件下，对试样未胶接端施加冲击力，使试样沿其胶接线产生剥离。韧性不同的结构胶粘剂，其剥离长度有显著差别，从中可判别出其韧性的优劣。

通过测量试样剥离长度以及对不同型号胶粘剂测试数据的比较分析，可制定出以剥离长度为指标的、简易、实用的结构胶粘剂韧性合格评定标准。

Second, the principle
1. A pair of mild steel sheets are glued into a T-impact stripping sample, and under the specified conditions, an impact force is applied to the unbonded end of the sample to cause the sample to peel off along the glue line. Structural adhesives with different toughness have significant differences in peel length, from which the toughness can be judged.
2. By measuring the peel length of the sample and comparing and analyzing the test data of different types of adhesives, a simple and practical structural adhesive toughness conformity assessment standard with the peel length as the index can be developed.

三、技术参数：

导杆：20mm

园钢杆22mm

冲击块:900g

钢杆长度：305mm

重量：25KG

Third, technical parameters
1. Guide rod: 20mm
2, garden steel rod 22mm
3. Impact block: 900g
4, steel rod length: 305mm
5, weight: 25KG