8000D Dual Mixer/Mill - Efficient two-clamp laboratory mill for pulverizing hard, brittle samples. - Also called a shaker mill or high-energy ball mill. - Sample sizes in the 10-gram per vial range. - Reduces samples to analytical fineness. - Can be utilized for mechanical alloying. - Features include a variable-range electronic timer, dual clamps for multiple sample capacity, safety interlock system, and forced-air cooling. - A European CEE 7/7 two-prong plug is supplied with the 8000D-230 systems. - One or two samples can be processed simultaneously, however if one sample is processed, a second empty vial must be inserted to maintain the balance of the system. - Accepts grinding/mixing vials from 2 3/8 in. to 3 1/8 in. in length and up to 2 1/4 in. in diameter. - The 8010 and 8011 adapters permit processing of multiple samples in 1/2 in. x 1 in., 1/2 in. x 2 in., and 3/4 in. x 2 in. vials.

8000D-230 Mixer/Mill
For 230 V/50 Hz operation. A European CEE 7/7 two-prong plug is supplied with the 8000D-230 systems.

| Specifications |  |
| --- | --- |
| Weight: | 82 lb. [37 kg], 98 lb. (44 kg) |
| Motor: | 1425 RPM [230 V/50 Hz] |
| Power Cord: | 2-prong European plug for 230 V/50 Hz version. |
| Clamp Movement: | 2 1/3 in. back-and-forth, 1 in. side-to-side |
| Clamp Speed: | 875 [230 V/50 Hz] |
| Voltage: | 230 V/50 Hz (4 amp fuse) |
| Controls: | Start, pause, and stop buttons; programmable timer with digital display, factory-set for 100 min. but extendible to 1000 minutes. |

8000M Mixer/Mill® - Also called a shaker mill or high-energy ball mill. - Sample sizes in the 10 gram range for grinding and up to 50 ml for mixing. - Reduces samples to analytical fineness. - Can be utilized for mechanical alloying. - Features include a variable-range electronic timer, safety interlock system, and shock-mounted electric motor for tabletop operation. - A European CEE 7/7 two-prong plug is supplied with the 8000M- 230 systems. - Accepts grinding/mixing vials from 13/4 in. to 31/8 in. in length and up to 21/4 in. in diameter. - The 8010 and 8011 adapters permit processing of multiple samples in 1/2 in. x 1 in., 1/2 in. x 2 in., and 3/4 in. x 2 in. vials.

8000M-230 Mixer/Mill
For 230 V/50 Hz operation. A European CEE 7/7 two-prong plug is supplied with the 8000M-230 systems

| Specifications |  |
| --- | --- |
| Dimensions: | 19 in. (48 cm) x 14 in. (36 cm) x 18 in. (46 cm) |
| Weight: | 76 lbs. net (35 kg); 92 lbs. gross (41 kg) |
| Motor: | 1/3 horsepower |
| Clamp Speed: | 875 [230 V/50 Hz] |
| Voltage: | 230 V/50 Hz (4 amp fuse) |
| Controls: | Start, pause, and stop buttons; programmable timer with digital display, factory-set for 100 min. but extendible to 1000 minutes. |

8001 Hardened Steel Vial Set Vial size 2-1/4 in. diameter x 3 in. Vial body and cap liner of hardened tool steel. Set includes screw-on cap with O-ring to allow wet or dry grinding/mixing, two 1/2 in. and four 1/4 in. steel balls. Grinding load 3–10 ml; mixing load approx. 25 ml. 8001B Steel Ball Set Two 1/2 in. and four 1/4 in. steel balls, for SPEX SamplePrep 8001 and 8009 vials.

8003 Alumina Ceramic Vial Set For dry grinding/mixing. Vial size 2-1/4 in. diameter x 2-3/4 in. High purity 99.5% alumina ceramic vial body and two slip-on caps with one 1/2 in. ball; set includes eight corprene gaskets. Grinding load 2–8 ml; mixing load approximately 20 ml. 8003A Alumina Ceramic Ball 1/2 in. diameter for 8003 vial. 8015 Clamp for 8003 Clamps permit slurry grinding in 8003 vial; sold in pairs. One pair required for each 8003.

8004 Tungsten Carbide Vial Set For wet or dry grinding/mixing. Vial size 2-1/4 in. diameter x 2-1/2 in. Tungsten carbide-lined body, two screw-on tungsten-carbide-lined caps, two 7/16 in. tungsten carbide balls, eight corprene gaskets. Grinding load 3–10 ml, mixing load approximately 25 ml. 8004A Tungsten Carbide Ball 7/16 in. diameter for SPEX SamplePrep 5004 and 8004 Tungsten Carbide Vials.

Grinding/Mixing Vial Dimensions. Vial P/N; Description; Overall Height w/Cap & Gasket (mm); Height w/o Cap (mm); OD (mm); ID (mm); Volume (ml). 3111; Polystyrene Vial; 27; 25; 13.6; 11.7; 2.5. 3114; Stainless Steel Vial; 27; 25; 12.7; 11.0; 2.5. 3116; Polystyrene Vial; 57; 51; 13.4; 10.7; 5. 3117; Hardened Steel Vial; 27; 25; 12.7; 11.0; 2.5. 3118; Agate Vial; 43; 33; 15.9; 9.5; 1.6. 3127; Hardened Steel Vial; 45; 42; 14.9; 12.4; 5. 5004; Tungsten Carbide Vial; 54; 48; 19.0; 12.5; 5. 6133; Polystyrene Vial; 54; 51; 20.1; 18.0; 12. 6133 PC; Polycarbonate Vial; 54; 51; 20.1; 18.0; 12. 6134; Polystyrene Vial; 79; 76; 27.9; 24.9; 35. 6135; Polystyrene Vial; 78; 76; 27.9; 24.9; 35. 8001; Hardened Steel Vial; 76; 64; 51.1; 37.8; 65. 8002; Polystyrene Vial; 72; 64; 53.3; 49.9; 135. 8003; Alumina Ceramic Vial; 70; 52; 51.1; 37.8; 45. 8004; Tungsten Carbide Vial; 62; 50; 50.7; 40.6; 55. 8003; Zirconia Ceramic Vial; 68; 52; 57.1; 38.0; 45. 8006; Methacrylate Vial; 64; 60; 50.9; 37.9; 55. 8007; Stainless Steel Vial; 76; 64; 50.8; 37.8; 65. 8008; Silicon Nitride Vial; 62; 52; 57.2; 37.5; 45. 8009; Round-End-Hardened Steel Vial; 76; 42; 58.8; 37.9; 35. 8014; Agate Vial; 70; 53; 46.9; 35.3; 40.

8010 Multiple-Sample Adapter Holds seven vials, each 1/2 in. in diameter, for simultaneous running in 8000M Mixer/Mill. Suitable vials include SPEX SamplePrep 3111, 3114, 3116 and 3117. Vials run at one time should be same length. Can be used in 8000D Mixer/Mill with 3116 vials, or with 3111, 3114, and 3117 vials if 8012 adapter is also used.

8011 Multiple-Sample Adapter. Holds four vials, each 3/4 in. in diameter, for simultaneous running in 8000M Mixer/Mill. Suitable vials include SPEX SamplePrep 3118, 3127, 5004 and 6133. Vials run at one time should be same length. Can be used in 8000D Mixer/Mill with 3118, 5004, and 6133 vials. To use 3127 vials in 8000D, 8012 adapter is also necessary.

8012 Vial Clamp Adapter for the 8000D Mixer/Mill Required when running 8010 adapter with 1 in. vials (3111, 3114, 3117) or 8011 adapter with 3127 vials in the 8000D.

3111 Polystyrene Vial 1/2 in. diameter x 1 in. long. Includes slip-on polyethylene cap. Capacity 2.5 ml; mixing load approx. 1 ml; accepts SPEX SamplePrep 3112 or 3119 plastic balls. Sold in units of 100.

3116 Polystyrene Vial 1/2 in. diameter x 2 in. long. Includes slip-on polyethylene cap. Capacity 5 ml; mixing load approx. 2 ml; accepts SPEX SamplePrep 3112 or 3119 plastic balls. Sold in units of 100. 3116PC Polycarbonate Vial 1/2 in. diameter x 2 in. long. Includes slip-on polyethylene cap. Capacity 5 ml; mixing load approx. 2 ml; accepts SPEX SamplePrep 3112 or 3119 plastic balls. Sold in units of 100.

6133 Polystyrene Vial 3/4 in. diameter x 2 in. long. Includes slip-on polyethylene cap. Capacity 12 ml; mixing load approx. 5 ml; accepts SPEX SamplePrep 3112 or 3119 plastic balls. Sold in units of 100. 6133PC Polycarbonate Vial 3/4 in. diameter x 2 in. long. Includes slip-on polyethylene cap. Capacity 12 ml; mixing load approx. 5 ml. Sold in units of 100. Accepts SPEX SamplePrep 3112, 3119, and 8006A plastic balls.

3114 Stainless Steel Vial Set 1/2 in. diameter x 1 in. long. Made of 303 stainless steel; includes slip-on cap and 1/4 in. stainless steel ball; grinding load 0.2–0.6 ml, mixing load 1 ml.

3127 Hardened Tool Steel Vial Set 3/4 in. diameter x 1 7/8 in. long. Made of Worpliss tool steel; includes center cylinder with two slip-on caps and 1/4 in. steel ball. Grinding load 0.5–1.0 ml, mixing load 2 ml.

3118 Agate Vial Set Handmade from flawless Brazilian agate; must be run inside cushioned SPEX SamplePrep 6133 vial (included). Includes stopper and 1/4 in. agate ball (3118A) Grinding load 0.2 – 0.6 ml, mixing load 1 ml.

5004 Tungsten Carbide-Lined Vial Set 3/4 in. diameter x 2 1/8 in. long. Includes two slip-on Delrin® caps with tungsten carbide inserts, six disposable methacrylate center cylinders, and two 5/16 in. tungsten carbide balls(5004A). Grinding load 0.5–1.5 ml; mixing load 3 ml.

窗体底部

窗体底部